Objective of the Day:

- Show 2 images and combine them essentially

  - 1 image is a face

  - 1 image is a texture

  - Can control the amount of overlap between the two

- Texture superimposed on the two triangles

- Because images are being used, he uses an open source library called SOIL

  - It is only for opening and manipulating images

  - Examples in class are only using open source libraries

  - SOIL is integrated in the Linux distribution itself

Discussion of Code:

- Mix_value is 0.2f	← Very important to note

- Initial set up is same as before in order to open the GL Window

  - All the code from Initial to the shader is essentially the same

  - Shader is changed a little bit

- Generate 2 texture tabs that we will later bind to the images on the triangle

- Generate 1st texture:

  - We do GenTextures: assigns texture to 1st image

  - Textures are 2D because the images are 2D

  - How to wrap the texture around the edges:

  - How to have the texture when there is no direct mapping

  - We are opening the image and doing the width and height of the image and having it load as an RGB

    - Take the image and convert into a texture map that we will later use

    - GL_TEXTURE_2D for the image

    - glGenerateMipMap → A mipmap is a hierarchical image that you can use at run time that determine how to run and load

  - After you will free image

  - After image is free then we will unbind the image

  - Bind the texture, specify the parameters, then unbind the texture

- Same code applies as is for the 2nd texture (the face)

- Specify how to map the images on the geometry

  - We have the positions and colors as previously done for each vertex

  - Now we do the textures for that as well

  - Indices array remains the same as we have 2 triangles

  - Buffer object remains the same except for the specification for the attributes

    - All of type float, and 3rd parameter is false, 4th parameter is of 8 because we need 8 floating point values before going onto the next vertex

    - Position starts at 0, 3 floating point values

      - Color starts at 1, 3 floating point values

      - Texture starts at 2, 2 floating point values

  - The Display Loop

    - Use our shader program

    - Bind the texture

      - activeTexture(GL_TEXTURE0)

        - First texture

    - Pass the mix_value to the shader

      - Used to change its behavior on how to display the data

- Shader.vs

  - Layout

    - Position vector3

    - Color vector3

    - Texture vector2

  - Output

    - Color

    - Texture coordinate

  - For texture coordinate, we do x, and then 1 - y coordinate

- Shader.frag

  - Input:

    - Our_color	//output of shader.vs

    - Texture coordinate //output of shader.vs

  - Output:

    - Color vector4

  - Uniform

    - Data type to allow the shader to communicate with the data

    - Lets the GPU talk directly to the main program without sending an array object

  - For our texture 1, we do the TexCoord

  - For our texture 2, we do 1-TexCoord.x, TexCoord.y

  - Mix_value changes based off the input from the keyboard

- Window Behavior

  - Escape key remains the same

  - Up key:

    - We will add 1

    - If goes at 1 we will stay at one

  - Down key

    - We will subtract 1

    - If goes below 0, we will stay at 0

Elements of a Circuit:

- LED:

  - had the anode → connect to positive

  - Cathode → connect to negative

- Resistor:

- Transistor:

  - If the base is off, no current can flow

  - The current flows from top to bottom

  - Max voltage supply in Arduino is 5V

  - If you want a larger voltage supply, then use a transistor

    - It will supply a low current through the transistor

    - Collector can be anything you want

      - Outer source, not from the Arduino

    - Main goal: Have large currents without frying the arduino

- Capacitor:

  - 2 Kinds of Capacitors that come with the kit, we will use the smaller one

    - 100nF is the one we will use

  - Slow down fluctuations in your circuit

  - Acts as a temporary power source

  - Purpose is to hold charge

    - Smaller capacitors hold smaller amount of charge

    - Larger capacitors hold larger amount of charge

- Switch

  - Four Legs with a Button on top that will close the circuit when pressed

  - Best way to put the switch on the breadboard is put it on the divider

    - May need to stretch the legs out a bit to fit

Assembly of the Circuit on the BreadBoard:

- Most of the time when operating with small elements it is best to use forceps

- Put the capacitor to the left of the switch

- Connect the ground supply to the same row as the switch(opposite of the board)

- Put the Resistor in parallel to the capacitor

- Connect the other leg of the switch to power supply(opposite of the board)

- 5V (blue wire) and Ground (yellow wire) from Arduino is connected to BreadBoard (Opposite of what the board says on the top)

- Connect the LED to Ground

- Connect the 1k Ohm Resistance in series with the LED

- Connect LED to pin 12

- Connect the Switch to Pin 7

- Code for the Circuit

  - Set LED to OUTPUT

  - Set Button to INPUT

  - In the loop

    - If the button is high (pressed)

      - Set the LED for High, delay for 500 (.5s) then set it back to low

- Behavior of the Code/Circuit

  - Press the button

  - The LED lights up for 0.5sec

  - The LED turns off